Modern Education Society's College of Engineering, Pune 411 001

Department of Mechanical Engineering

| Name |  | Date of Performance |
| --- | --- | --- |
| Roll No. |  | Date of Submission |
| Branch |  | Examined |

# Experiment No:

## Title: Linear Measurement by Vernier Calliper, Digital Vernnier Calliper and Micrometer

## Apparatus:

| Instrument | Specifications |
| --- | --- |
| Vernier Calliper | L.C. 0.02 mm |
| Digital Vernnier Calliper | L.C. 0.01 mm |
| Micrometer | L.C. 0.01 mm |

## Introduction:

Linear measuring instruments are designed either for linear measurement (i.e. measuring distance between two lines) or end measurement (i.e. measuring distance between two surfaces).

Line graduated instruments have graduations engraved on the parts of the instruments and distance between these graduations is known. They are used for direct measurement. Measuring accuracy depends on original accuracy of graduations, low efficiency, the instrument is read, definition of graduation lines and general design and workmanship of the device.

Mainly two types:

- Non-precision instruments

- Precision instruments

If the error in the dimension is more than 0.25 mm the instrument producing it are called Non-precision instruments. Precision instruments produce error as less as 0.0025 mm or less in measured dimensions.

A precision measuring instrument should have:

- High degree of sensitivity and accuracy.

- Minimum inertia of moving parts of mechanisms.

- It should eliminate the possibility of error in transferring accurate measurements.

- It should be self-checking and should have good precision.

- Abbes principle should be followed in the design.

## VERNIER CALLIPER:

The principle of vernier is that when two scales or division slightly different in size are used, the different between them can be utilized to enhance the accuracy of measurement.

## DISCRIPTION:

The vernier calliper essentially consists of two steel rules and these can slide along each other. The main scale is engraved on a solid L-shaped frame. On this scale graduations are divided into 20 parts so that one small division equals 0.05 mm. One end of the frame contains a fixed jaw, which is shaped into a contact tip as its extremity. The sliding jaw at its left extremity contains another measuring tip. When two measuring tip surfaces are in contact with each other, scale shows zero reading. The adjusting screw can do the finer adjustment of the movable jaw. The locking nuts are adjusted before taking the final reading. The measuring tips are so designed as to measure inside as well as outside dimensions.

## LEAST COUNT:

The difference between one division on main scale and vernier scale represents the accuracy to which the reading may be taken. This is known as least count of vernier.

Least Count (L.C.) = Smallest division of main scale / Total no. Of divisions on vernier scale

If the smallest division on main scale is 1 mm and there are 50 divisions on vernier scale then,

L.C. = 1/50 = 0.02 mm

Total reading on vernier calliper = (Reading on main scale) + (L.C. × mark no. of division on vernier scale that coincides with any mark on main scale)

## MICROMETER:

Principle: It consists of employment of a screw and nut, both having accurately cut threads. One complete revolution of the screw will advance it, in relation to the nut threads, a distance equal to lead of the thread.

Construction: Five principle parts 1) Frame 2) Anvil 3) Spindle 4) Sleeve 5) Thimble.

1) Frame: The frame is U shaped, at the left end of which an anvil is attached and to the right end, a barrel or sleeve is attached.

2) Anvil and spindle: Fixed anvil is at left hand and is screwed in frame. Measuring faces are of hardened steel. The anvil should be ground and lapped flat and parallel to face of spindle.

3) Thimble and barrel: The micrometer screw rotates in a fixed nut, which is press, fitted into barrel, which is integral part of frame. The screw on rotations moves in an axial direction relative to the nut. To this screw, spindle is directly attached and the opposite end screw is rigidly attached to thimble. So when thimble rotates and moves axially along the barrel when screw is rotated,

4) With main nut, another short nut is attached to it having springs between them to avoid blacklash in screw and nut and these two nuts are held by serration like clutch. The shorter nut is rotated and displaced w.r.t. main nut by adjustment for wear of threads.

Least Count (L.C.) = Smallest division of main scale / Total no. of divisions on circular scale

Reading on micrometer = Reading uncovered on barrel + (L.C. × no. of divisions on thimble scale which coincides with horizontal line on barrel)

For vernier calliper:

L.C. = Smallest division on main scale/No. of division on vernier scale = 0.5 /25 = 0.02 mm

For micrometer:

L.C. = Pitch of spindle screw / No. of division on thimble = 0.5 / 50 = 0.01 mm

## OBSERVATION TABLE:

A) For Vernier Calliper

| Sr. No. | Dimension | Reading 1 | Reading 2 | Reading 3 | AVERAGE |  |
| --- | --- | --- | --- | --- | --- | --- |

B) For Digital Vernier Calliper:

| Sr. No. | Dimension | Reading 1 | Reading 2 | Reading 3 | AVERAGE |  |
| --- | --- | --- | --- | --- | --- | --- |

C) For micrometer:

| Sr. No. | Dimension | Reading 1 | Reading 2 | Reading 3 | AVERAGE |  |
| --- | --- | --- | --- | --- | --- | --- |

## Result Table:

| Sr. No | Vernier Calliper | Digital Vernier Calliper | Micrometer |
| --- | --- | --- | --- |

## CONCLUSION:

Vernier calliper and micrometer as precision instruments are studied.

## QUESTIONS:

- What is the difference between precision and non-precision instruments?

- Write the procedure of calculating Least Count of Vernier calliper and micrometer.

- What is the principle of Vernier calliper / Micrometer?

- State the errors during measurement by Vernier calliper.

- What is depth micrometer?